SPECIFICATIONS – BIRD·B·GONE POLYCARBONATE BIRD SPIKES

PART 1 – GENERAL

  - SUMMARY

- Design Requirements: Select appropriate size as determined by site conditions to prevent birds from landing and roosting on specified surfaces.

  - QUALITY ASSURANCE

- Single Source Responsibility: Furnish products from one manufacturer for entire project.

- Country of Origin: Country of Origin shall be designated at U.S.A.

- Obtain all technical information from the manufacturer.

- Utilize labor or Bird·B·Gone® Authorized Installers who are knowledgeable in Bird·B·Gone product installations.

- Installer shall visit the site to gather all information of existing site conditions.

  - SUBMITTALS

- Product Data: Submit manufacturer’s descriptive literature and product specifications for each product including catalogs, installation instructions and other descriptive material.

- Provide Warranty: Material and installations.

- Provide Samples: Each type of spike excluding proposed fastening methods.

- Provide statement by official indicating that they are a certified installation company.

  - PRODUCT HANDLING

- Protect Bird·B·Gone products from damage before, during and after the installation.

  - PROJECT CONDITIONS

- Coordination: Furnish samples of system(s) so installation can be coordinated with existing conditions and within on-site conditions.

- Visit site and field measure prior to fabrication and delivery of materials.

  - WARRANTY

- Product shall carry a minimum 5-year guarantee against manufacturer’s defect and U.V. breakdown.

PART 2 – PRODUCTS

  - ACCEPTABLE MANUFACTURER

Manufacturer:

Bird B Gone LLC

Bird·B·Gone Polycarbonate Bird Spike

1921 E. Edinger Ave.

Santa Ana, CA 92705

Tel: 800-392-6915 or 949-472-3122

Fax: 949-472-3116

P.O.C.: Chris Fields

  - PRODUCT DESCRIPTION

- Model Designation:

  - Bird·B·Gone Polycarbonate Bird Spike - 3”, 5” or 7”

    - Width of Coverage:

i.    BBG2000-3: 1” – 5” widths

ii.   BBG2000-5: 5” – 7” widths

iii.  BBG2000-7: 6” – 9” widths

    - Height: 4-1/2”

    - Base: Flexible

i.  Glue trough feature (underside) for ease of attachment with adhesive.

ii.  Offset pre-drilled holes, for ease of attachment with screws and / or adhesive.

    - Length: 2 ft. sections

    - Packaging: 50ft. per box. Twenty-five (25) 2ft. sections per box.

    - Color: As designated by the project specifications.

    - Number of Rows: As determined by the manufacturer and based on project conditions.

    - Mounting System: As determined by the manufacturer and based on project conditions.

- Product made in the U.S.A.

  - MATERIALS

- Material: U.V. stabilized Polycarbonate.  Heat and weather resistant (+310°F to –200°F).  Non-conductive and will not interfere with electronic security systems or transmissions.

- Construction: Rigid one piece construction, available in 7 standard colors including crystal clear.

- Adaptation: “Break away” points are located between each set of screw holes.  Tin snips, hacksaw, etc. can also be used to cut the product to the desired length.

  - MOUNTING SYSTEMS

- Wood: Use #8 Wood Screws.  For every 2-ft. section of Bird·B·Gone Polycarbonate Bird Spike ten (10) #8 wood screws are required.

- Steel, Brick, Stone or Concrete: Use an outdoor construction adhesive that is non-silicone based.  Purchase from the manufacturer or call for recommended adhesives.  If mounting surface warrants, screw or bolt down Bird·B·Gone Polycarbonate Bird Spike in conjunction with using the adhesive.

- Pipes or Poles: Ty-Wraps can be used alone or in conjunction with an outdoor construction adhesive.  For U.V. protected Ty-Wraps, purchase directly from the manufacturer or call for recommended products.

PART 3 – EXECUTION

  - EXAMINATION

- Examine the installation area and note any detrimental or hazardous work conditions.  Notify contracting officer or inspector of the detrimental work conditions.

- Do not proceed with installation until conditions are corrected.

  - SURFACE PREPARATION

- Surface should be thoroughly cleaned and free of bird droppings, nesting materials, rust, peeling paint or other debris.

- Remove or repair articles that may damage Bird·B·Gone Polycarbonate Bird Spike after installation, including overhanging foliage, brush and loose parts on the structure.

  - INSTALLATION

- Install Bird·B·Gone Polycarbonate Bird Spike as recommended by the manufacturer.

- Bird·B·Gone Polycarbonate Bird Spike should be installed correctly, covering the entire depth of the surface, not just the perimeter.

  - Follow the contours and angles closely: cut or break away to fit properly.

  - Space materials in accordance with manufacturer’s recommendations.

  - INSPECTION

- Visually inspect Bird·B·Gone Polycarbonate Bird Spike for any signs of poor installation, including loose screws, fasteners and un-removed debris.

- Immediately correct and repair as necessary.

END OF SPECIFICATONS